# Your Solar Design Pack — Project Workbook

Capstone project for “Solar Installation: Design, Install and Commission Home Systems Safely” · Trades · Beginner-friendly; no electrical experience needed to start · Skillnaija (free course, verifiable certificate)

Pass mark: 70% · Submit at www.skillnaija.org → the course → “Submit your project” · A human examiner also reviews this safety-critical project.

# How to use this workbook

Certificates on Skillnaija are earned by passing this project, not by finishing the lessons. This workbook is the whole brief in one file so you never have to search for what is required.

- Work through every step below and write your answers under it (type here, or print and write by hand, then photograph).

- Keep evidence as you go: photos, screenshots, spreadsheets, receipts, links. Tick the evidence checklist before you submit.

- Score yourself against the rubric at the end. If you would not give yourself the pass mark, improve the weak part first — you can only resubmit after a fail, not after a pass.

- Then upload this workbook (and your evidence) to Google Drive, make the link PUBLIC using the steps at the end, and paste the link into the submission box together with your write-up.

# Your brief — what to submit

One shared link (a Google Doc, or a folder with view access) containing every part, with photos and the diagram (hand-drawn and photographed is fine). Do not include the client's address or personal details beyond the town.

# What to do, step by step

## Step 1 of 7

Part 1. Survey: choose a real home or small business (yours, a relative's or a client's, with their permission). Record the roof (type, condition, orientation, shade between 9 am and 4 pm), where the battery and inverter would go, cable routes, the distribution board and the earthing, with photos. Do not climb a roof for this project: survey it from the ground, a window or a safe vantage point.

Your work:

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

Evidence for this step (file name, photo, link):

_____________________________________________________________________

_____________________________________________________________________

## Step 2 of 7

Part 2. Load audit: list every appliance to run on solar with its watts, quantity and hours per day, and the total Wh per day. Say which loads stay off solar and why, and what the client could change to use less. Separate night-time use from daytime use.

Your work:

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

Evidence for this step (file name, photo, link):

_____________________________________________________________________

_____________________________________________________________________

## Step 3 of 7

Part 3. Sizing, with every step of the working shown: add losses; the array size from your location's peak sun hours and a 0.8 derate; the panel you chose (with its datasheet values) and how many; the string design, checking Vmp against the MPPT window and cool-morning Voc against the maximum input voltage; the battery bank from night-time use and the depth of discharge for your chosen chemistry; and the inverter from the largest simultaneous load, the margin and any motor start surge.

Your work:

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

Evidence for this step (file name, photo, link):

_____________________________________________________________________

_____________________________________________________________________

## Step 4 of 7

Part 4. Single-line diagram: from the panels to the house board, showing the DC isolator, surge protection, battery fuse or breaker with its rating and position, inverter, changeover, AC protection (MCB and 30 mA RCD) and earthing. Label every cable with its size, and show your voltage-drop sums for the PV run and the battery run.

Your work:

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

Evidence for this step (file name, photo, link):

_____________________________________________________________________

_____________________________________________________________________

## Step 5 of 7

Part 5. Bill of materials and quote: every item with its specification and a current price you collected (name where you got it; three quotes for the big items if you can). Add labour, transport, contingency, warranty terms and payment milestones. Say how you checked that the main items are genuine.

Your work:

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

Evidence for this step (file name, photo, link):

_____________________________________________________________________

_____________________________________________________________________

## Step 6 of 7

Part 6. Installation method statement: the order of work and, for each hazardous step (working at height, DC connections, battery connection, AC connection), the safety controls: isolation, test-before-touch, PPE, fall protection and who supervises. State who the qualified, certified installer responsible for the work would be.

Your work:

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

Evidence for this step (file name, photo, link):

_____________________________________________________________________

_____________________________________________________________________

## Step 7 of 7

Part 7. Commissioning and handover: a commissioning checklist with the expected readings (for example string Voc) and the start-up and shut-down sequence, plus a one-page handover sheet for the client in plain words.

Your work:

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

Evidence for this step (file name, photo, link):

_____________________________________________________________________

_____________________________________________________________________

# Evidence checklist — tick before you submit

| Evidence | Included? (✓) | Where is it (file / link / page) |
| --- | --- | --- |
| Step 1 — work and proof |  |  |
| Step 2 — work and proof |  |  |
| Step 3 — work and proof |  |  |
| Step 4 — work and proof |  |  |
| Step 5 — work and proof |  |  |
| Step 6 — work and proof |  |  |
| Step 7 — work and proof |  |  |
| The final deliverable described in the brief |  |  |
| Your write-up (what you did, choices, what you learned) |  |  |
| Link tested in an incognito window — opens without sign-in |  |  |

# How it is graded — score yourself first

The examiner scores each criterion 0–100 and weights them as below. You need 70% overall to pass.

| Criterion | Weight | My honest score (0–100) |
| --- | --- | --- |
| Survey and load audit are real, complete and honest, with night and day use separated | 15% |  |
| Sizing is correct with the working shown: losses, array, string limits, battery depth of discharge and inverter surge | 25% |  |
| The single-line diagram is correct and safe: DC-rated protection in the right places, battery fuse at the battery, cable sizes with voltage-drop sums, earthing, RCD and changeover | 25% |  |
| The bill of materials and quote are realistic, with genuine-equipment checks | 10% |  |
| The method statement controls every serious hazard and names supervision by a qualified, certified installer | 15% |  |
| Commissioning checklist and handover sheet are clear and usable | 10% |  |

A useful habit: read each criterion and ask “where in my submission would an examiner SEE this?” If you cannot point to it, it is not there yet.

# Your write-up — paste this into the submission box

Keep it under 5,000 characters. Three short parts:

## 1. What I did

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

## 2. The choices I made and why

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

## 3. What I learned and what I would improve next time

_____________________________________________________________________

_____________________________________________________________________

_____________________________________________________________________

# Before you submit — make your link PUBLIC

Learners fail this project for one avoidable reason: the link they paste is private. The examiner and any employer who later views your certificate cannot “request access” — a link that asks for sign-in counts as no submission.

- Google Drive (file or folder): right-click the file → Share → under "General access" change "Restricted" to "Anyone with the link" → role "Viewer" → Copy link. Then open the link in a private/incognito window: if it asks you to sign in or "Request access", it is NOT public yet.

- Google Docs, Sheets or Slides: Share (top right) → "Anyone with the link" → Viewer → Copy link. Same incognito test.

- Dropbox: Share → Create link → make sure it says "Anyone with this link can view".

- OneDrive: Share → "Anyone with the link can view" → Copy link.

- Canva: Share → "Anyone with the link" → Can view → Copy link.

- GitHub: the repository must be Public (Settings → Danger zone → Change visibility). Private repos cannot be opened.

- YouTube or video: "Unlisted" is fine; "Private" is not. WhatsApp links and files sent by chat cannot be graded.

- Photos: put them in one Drive folder shared as above and paste the FOLDER link — not one photo per link.

Final test: paste your link into a private/incognito browser window. If the work opens with no sign-in, you are ready.

© Skill Naija LTD · www.skillnaija.org/#/project/solar-installer · This workbook is free; the certificate is earned.